Transducers for M8

|  | Probe | Illustration | Parameters | Application | Biopsy kit |
| --- | --- | --- | --- | --- | --- |
| Convex | C5-1s |  | Bandwidth: 1.3-5.7 MHz ExFOV: 101° Radius: 60mm | Adult Abdomen Gynecology Obstetrics | NGB-022, multi angle, reusable |
|  | C6-2Gs |  | Bandwidth:1.7-6 MHz ExFOV:132° Radius:20mm | Interventional | NGB-024, multi angle, reusable |
|  | D7-2s |  | Bandwidth: 1.6-7.2 MHz FOV: 71° X 70°(sweep) | Obstetrics | None |
| Micro-convex | C11-3s |  | Bandwidth:3-11.2 MHz ExFOV: 121° Radius: 15mm | Abdomen Pediatrics Transcranial Vascular Small parts Musculoskeletal | NGB-018, multi angle, reusable |
| Endocavity | V11-3Ws |  | Bandwidth:3-11.2 MHz ExFOV: 180° Radius: 11mm | Gynecology Obstetrics Urology | NGB-004, single angle, reusable |
| Linear | L10-3s |  | Bandwidth:2.7-10.5 MHz FOV:34mm | Vascular Small parts Musculoskeletal Nerve | Not available |
|  | L13-3s |  | Bandwidth: 3.2-12.3MHz FOV: 37.9 mm | Small parts Vascular Pediatrics Superficial Musculoskeletal Neurology Abdomen | NGB-007, multi angle, reusable |
|  | L14-6Ns |  | Bandwidth:3.5-16 MHz FOV:38mm | Small parts Vascular Pediatrics Superficial Musculoskeletal Neurology | NGB-007, multi angle, reusable |
|  | L16-4Hs |  | Bandwidth:3.5-16 MHz FOV:25mm | Musculoskeletal Nerve Intra-operative Vascular Surgery | Not available |
|  | LM14-6s (Matrix) |  | Bandwidth:3.5-16 MHz FOV:38mm | Small parts Musculoskeletal Nerve | NGB-023, multi angle, reusable |
|  | L14-6Ws |  | Bandwidth:3.5-16 MHz FOV:50mm | Breast Small parts Musculoskeletal Nerve | NGB-007, multi angle, reusable |
|  | L20-5s |  | Bandwidth: 6-23 MHz FOV: 29mm | Musculoskeletal | None |
| Sector | P4-2s |  | Bandwidth: 1.3-4.7 MHz FOV:90° | Adult Cardiac Transcranial Adult Abdomen | NGB-011, multi angle, reusable |
|  | P10-4s (High Density) |  | Bandwidth:2.9-10.5 MHz FOV: 90° | Neonatal Cardiac Transcranial | Not available |
|  | P7-3s |  | Bandwidth:2-8 MHz FOV: 90° | Pediatric abdomen Pediatric cardiac Neonatal cephalic Neonatal abdomen Neonatal cardiac | Not available |
| TEE | P7-3Ts |  | Bandwidth:1.9-8.2MHz FOV:90° | Adult cardiac | Not available |
|  | CW2s |  | Bandwidth: 2MHz | Cardiac | Not available |
|  | CW5s |  | Bandwidth: 5MHz | Vascular | Not available |